Chapter 1

Section 1

The effectiveness of data communication is based on three functionalities. Delivery, Timeliness and Accuracy. Delivery – the system must deliver data to the correct destination. Timeliness – must be delivered in a timely matter. Accuracy – it must be delivered correctly.

A network is a set of nodes connected by media links. There are three criteria for data communication, Performance, Reliability, and security.  Performance is measured in transit time and response time. Reliability of network includes the accuracy of the delivery, failure frequency and recovery time. Security is protecting data.

Data Flow:

Section 2

Types of Connections:

There are 4 different types of topology. Mesh Star Bus and Ring.

Mesh:						Advantages:

Each link carries its own data.

Its robust, so if one goes down the others stay up

Private, only intended receiver sees the content

Disadvantages:

Installation and reconfiguration are difficult

Hardware required is expensive.

Lots of cabling and I/O ports needed.

Star:						Advantages:

Robust

Easy fault identification and isolation

Disadvantages:

Cabling amount

Bus:

Advantages:

Installation is easy

Disadvantages:

Reconfigurations and fault isolation are difficult

Ring:

Advantages:

Installation and reconfiguration are easy

Faults isolation is simplified (alarm alert for signal lack)

Dual ring or switches are used to overcome ring break

A hybrid topology is possible to have, meaning there can be two topologies in one.

There are types of network. Local area network (LAN) , Metropolitan area network (MAN), Wide area network (WAN).

Single building LAN:			MAN:

WAN:

Chapter 2

Section 1 Layered Tasks

We use layers to break things down.

Section 2 OSI Model

The OSI model is designed with 7 layers. These layers help with the process of transferring data. Thee layers are as followed in descending order:

7.Application

6.Presentation

5.Session

4.Transport

3.Network

2.Data link

1.Physical

Section 3 Layers in the OSI

Physical Layer

Physical Layer is responsible for the movements of individual bits from one node to the next.

Concerned with physical characteristics of interface and media, representation of bits, data rate, synchronization of its, line configuration, physical topology, transmission mode.  To transmit bis over a medium to provide mechanical and electrical specifications.

Data Link Layer

Responsible for moving frames from one node to the next. Concerned with framing,physical addressing, flow control, error control, access control. To organize bits into frames; to provide node to node delivery.

Responsibilities:

• Framing, dividing messages into smaller sets of data.

• Responsible for sending the frames over the network.

• Frames should be made transmittable, can go through network without a need to change the data within.

• Physical Addressing. It gives the frame the physical address of the next node, not the final destination.

• Flow Control

• Error Control

• Access Control

• Data link layer example:

• Hopping from node to node is a function of the data link layer.

T2(trailer) Data 10(source address) 87(dest address)

Network Layer

Responsible for the delivery of individual packets from the source host to the destination host. Concerned with logical addressing and routing. To move packets from source destination; to provide internetworking.

• Communicating device: Any device capable of sending and receiving data.

• Connecting device: Connects different devices together.

Both of these are nodes.

• While hop-to-hop is handled by data link layer, the overall END to END is taken care of by the network layer.

Responsibilities of the Layer:

• Logical Addressing

• Routing

During routing, the physical addresses of the packet change all the time as the packet bounces between nodes. However the logical addressing does not ever change.  The routing table in each router makes direction decisions.

Transport Layer

Responsible for the delivery of a message from one process to another. Concerned with service-point addressing, segmentation and reassembly, connection control, flow control, and error control. To provide reliable process to process message delivery and error recovery.

The transport layer is responsible for the delivery of a message from one process to another.

Responsibilities:

• Service-point addressing. Essentially Port numbers of the running programs that are bound to sockets.

• Segmentation and Reassembly. Sometimes you need to break a message down during transport.

• Connection Control

○ Instead of sending all packets through one route, it organizes it over multiple routes.

○ Flow control: The receiver should be able to receive the data the same way it was sent. On transport layer, it handles flow control from PROCESS TO PROCESS, not NODE TO NODE(which is handled by another layer.

• Error Control:

Session Layer

Responsible for dialog control and synchronization. Concerned with dialog control and synchronization. Establishes manage and terminate sessions.

Responsible for dialog contrl and synchronization

Responsibilities:

• Dialog control: whether it is set to duplex, simplex, half-duplex.

• Synchronization: allows you to have checkpoints in data transmission that allow you not to have to re-send the whole message if something goes wrong.

Presentation Layer

Responsible for translation encrypt and compress data.

Responsible for translation, encryption.

Responsibilities:

• Translation: translating between guided and unguided media.

○ Sidenote: converting from bitstream to signal is physical layer

○ Converting rom Application to Bitstream is the PRESENTATION LAYER.

○ Converts application-specific encoding to generic encoding. This generic encoding is passed to receiver, where the presentation layer decodes it back to the application specific encoding.

• Encryption:

○ Changes form of data to another form that is encrypted.

Application

Responsible for providing services to the user. Concerned with network virtual terminal, file transfer/access/management(FTAM), mail services and directory services. To allow access to network resources.

Responsible for providing services to the user

• Network virtual terminal

• He went way too fucking fast.

Section 4 TCP/IP Protocol Suite

It has 4 layers, Host-to-host network, internet, transport and application. If you compare it to the OSI it could have five layers where host to host network is broken up into Physical and Data Link.

Uses a 6-byte with 2 hex digit bits for the physical address. Physical address wil change from node to node but he logical address remains the same.

Chapter 8

Section 1

A single bit error only is when only 1 bit in the data has changed. A burst error is when 2 or more bits have changed.

Section 2

In block coding we divide our message into blocks, each of k bits called dataword. We add r redundant bits to each block to make the length n = k+ r. The resulting n-b9it blocks are called codewords.

The hamming distance between two bits is the number of one digits after they are XORd together. For example 000 XOR 011 is 011 therefore the hamming distance is 2. The minimal hamming distance is the smallest value of all combinations of pairs.

Section 3

All block codes belong to a subset called linear block codes. A linear block code is a code in which the XOR of two valid codewords creates another valid codeword.

Section 4

Cyclic codes are special linear block codes with one extra propterty. If a codeword is cyclically shifted(rotated), the resut is another codeword.

Checksum

Sender Side

Take Packet Split into 4 bit sections

Add them all together

Anything that exceeds 4 bits gets wrapped around and added again

Take ones compliment and that gives ur checksum that the receiver would use

Receiver Side

Take the packet and split into 4 bit sections again

Add them all together

Instead of wrapping around you add the checksum

Then wrap around and add

Takes ones compliment of that and it should be all zeroes if no errors.

Chapter 11

V Data link layer packs bits into frames.

A frame will look like the following:

Byte stuffing is the process of adding a extra byte to where an indicated flag is or escape character. Bit stuffing is adding an extra zero after 5 consecutive 1s.

Most important of datalink is flow control and error control, these are known as data link control.

Flow control is the set of procedures used to restrict the amount of data that the sender can send before waiting for acknowledgment. Error controls is based on ARQ (automatic repeat request), which is the transmission of data.

Noiseless stop and wait

Stop and wait is the same for both noiseless and a noisy channel, except in the acknowledgement it puts a sequence for the ARQ/noisy.

In the Go-Back-N Protocol, the sequence numbers are modulo 2^m -1, where m is the size of the sequence number field in bits. There is a sending window, if the sender does not receive acknowledgement for the frames that have been sent and the sender reaches the end of the sending window it will then restart at the beginning of the window.

Selective repeat arq, sends the frames one after the other, If the receiver for some reason receives a frame that is not in the sequence order it will send back a NAK which will then notify the sender that something messed up and it will restart at  the lost frame.2 ^(m-1)

HDLC – High level data link controlis a bit-oriented protocol for communication over point-to-point and multipoint links.

FCS – Frame control sequence, which referes to the extra checksum.

Chapter 19

Section 1

IPv4 addresses are 32 bits long.

How to determine what class an IP address falls under is given by the first couple digits. The following picture explains it:

a.   00000001 00001011 00001011 11101111

b.   11000001 10000011 00011011 11111111

c.   14.23.120.8

d.   252.5.15.111

Solution

a. The first bit is 0. This is a class A address.

b. The first 2 bits are 1; the third bit is 0. This is a class C
     address.

c. The first byte is 14; the class is A.

d. The first byte is 252; the class is E.

CIDR refers to which digits are part of the host ID.

In IPv4 a block of addresses can be defined as x.y.z.t/n

Chapter 20

Section 1

Switching at the network layer in the internet uses the datagram approach to packet switching

Communication at the network layer in the internet is connectionless.

Section 2

IPv4 is a delivery method for the TCP/IP protocol

Position of IPv4 in TCP/IP

IPv4 datagram format

The total length field defines the total length of the datagram including the header.

Version is 4 bits either 4 or 6 if not then just drop it.

Hlen length convert the bits to decimal then multiply by 4 bits.

Service Types  or Differentiated Service

For service types , 8 bits  only one can be set. If more than one it gets dropped aswell

Total length, 16 bits long.

Identification which is basically just the sequence number.

Flags is 3 bits:

Fragmentation Offset:

TTL: Time to live: essentially just a counter which jut means it can go through however many routers. If it goes through to many it gets dropped.

Protocol :